3.3 Spreadsheet Data Entry

Once the user presses the <ESC> key, the main spreadsheet will be displayed on the screen.  The user should proceed to put in values for each cell in the watershed.  Each column corresponds to one cell in the watershed.  The main spreadsheet is displayed in Figure 3-2 below.

<F1>-Help  <F2>-Menu  <F3>-Load  <F4>-Save  <F5>-Quit  <F6>-Initial Data

Cell Number...........              1             2             3            4           5          6            7

Cell Division.........             000        000          000        000       000      000         100

Rec. Cell Number......           4            1              7           7           4          5             7

Rec. Cell Div.........            000        000          220        220       000      000        230

Flow Direction........               5            7             5           6            7          7            3

SCS Curve Number......      74          74           74          74         78         82        100

Land Slope (%)........         0.9          0.8          0.8         1.1        1.1        0.8         0.0

Slope Shape...........              1            1             1            1          3            1            1

Slope Length (ft).....          100        100         100        100       100        100            0

Overland Manning's....   0.103     0.130      0.103      0.027    0.071     0.130     0.990

K - Factor............            0.28       0.28        0.32       0.32      0.30       0.29       0.00

C - Factor............        0.5000   0.4300    0.5100   0.5000   0.6000   0.4300   0.0000

P - Factor............            1.00       1.00       1.00       1.00       1.00       1.00       0.00

Surf Cond Constant....     0.29       0.29        0.21      0.23       0.12       0.29       0.00

COD Factor............           86           80        110          88        125         80            0

Soil Texture #........              2             2            2           2             2           3            0

Fertilizer Indicator..              1             1            1           1             1           1            0

Pesticide Indicator...            0             0            0           0             0           0            0

Point Source Ind......           0              0            0           0             0           0            0

Additional Erosion....           0              0            0           0             0           0            0

Impoundment Ind.......         0              0            0           0             0           0            0

<F7>-Go To  <F8>-Copy Cell  <F9>-Copy Row  <F10>-Search Row  <AltF1>-Clear

<AltF2>-Delete Cell  <AltF3>-Divide Cell  <AltF4>-Status  <AltF5>-Print

0FIGURE 3-2.  AGNPS Spreadsheet

The Cell number is the main identifier for each cell in a watershed. Each cell in the watershed is identified by a number. The cells are numbered consecutively, beginning at the cell in the northwest corner, and sweeping from west to east, north to south.  These numbers are automatically put into the spreadsheet.  If the user divides a cell, the computer automatically will show the resulting cells of this division on the spreadsheet.  The cell number will stay the same, but there will also be a number in the cell division line corresponding to the division.  An example of the numbering scheme for a divided cell is shown in Figure 3-3.  More information on cell division can be found in Chapter 7 of this manual.

The Receiving cell number is the number of the cell into which the most significant portion of the runoff from a cell drains. The receiving cell number for the watershed outlet must be a number greater than the total number of cells in the watershed.

The Receiving Cell Division is the number of the divided cell which receives the most significant portion of the runoff from the cell above.  Each division of a cell is one quarter of the area of its base cell.  An example of the numbering scheme is shown in Figure 3-3.  Chapter 7 of this user guide will give more information about cell division.

Base Cell #3

0      FIGURE 3-3. Cell Division Numbering Scheme

3 100

3 210

3 221

3 222

3 223

3 224

3 233

3 234

3 310                3 320

3 330                3 340

3 400

The Flow Direction is determined by cell topography and/or channel flow.  Flow Direction is a single digit in the range of 1-8 indicating the principal direction of drainage from the cell.  Each value refers to a direction with 1 representing north and, proceeding clockwise, 8 representing northwest.  See Figure 3-4 below.  In the example in Figure 3-5, the flow direction of cell 2 is to the west and is designated by the number 7.  The flow directions for cells 1 and 3 would both be 5, because the flow path is to the south.  Even numbered aspects only apply when the current cell and the receiving cell touch at a corner!

1

8                          2



                   

7                       3

                   



6                           4

5

0Figure 3-4.  Flow Direction

If there is no drainage from the cell, i.e. a sinkhole cell, a value of 0 for flow direction should be input.

0Figure 3-5.  Example Watershed

The SCS Curve Number or hydrologic soil-cover complex number is used in the Soil Conservation Service equation for estimating direct runoff from the storm rainfall.  Although the curve number for any antecedent moisture condition may be used, values for moisture condition II (average moisture) are listed in table 3-1 below.  For moisture conditions I (dry) or III (wet), equivalent curve numbers are shown in table 3-2.  Just look up the proper number in table one and then apply this to table 3-2 to find the appropriate value.  If more than one land use code exists within a cell, use a weighted average value if possible.

Land Use                      ----- Soil Group ----

A       B       C      D

Fallow                       77    86    91    94

Row Crop-Straight            67    78    85    89

Row Crop-Contoured           65    75    82    86

Small Grain                  63    74    82    85

Legumes or rotation meadow   58    72    81    85

Pasture-Poor1                68    79    86    89

Pasture-Fair2                49    69    79    84

Pasture-Good3                39    61    74    80

Permanent Meadow             30    58    71    78

Woodland                     36    60    73    79

Forest with heavy litter     25    55    70    77

Farmsteads                   59    74    82    86

Urban (21-27%) Impervious    72    79    85    88

Grass Waterway               49    69    79    84

Water                       100   100   100   100

Marsh                        85    85    85    85

Animal Lot-Unpaved           91    91    91    91

Animal Lot-Paved             94    94    94    94

Roof Area                   100   100   100   100

0TABLE 3-1.  Runoff Curve Number for Antecedent Moisture Condition II

SOURCE: U.S. Dept. of Agricultural, Soil Conservation Service (1976)

1  Pasture should be considered poor if it is heavily grazed with no mulch.

2  Fair pasture has between 50-75% plant cover and is moderately grazed.

3  Good pasture is lightly grazed and has more than 75% plant cover.

CN        Adjusted CN    Adjusted CN

for            for            for

Condition II   Condition I   Condition III

94		85	98

91		80	97

89		80	97

86		72	94

85		70	94

84		68	93

82		66	92

81		64	92

80		63	91

79		62	91

78		60	90

77		59	89

75		57	88

74		55	88

73		54	87

72	53	86

71	52	86

70	51	85

69	50	84

68	48	84

67	47	83

65	45	82

63	43	80

61	41	78

60	40	78

59	39	77

58	38	76

55	35	74

49	30	69

39	21	59

36	19	56

30	15	50

25	12	43

0TABLE 3-2. Adjusted CN Numbers for Types I and III

SOURCE: U.S. Department of Agriculture, Soil Conservation Service (1972)

The Land Slope for the cell is input by the user as a percentage.  This can be obtained from topographic maps, existing data or actual measurements. Assume an average slope if the cell is irregular.  If the cell is predominantly water or marsh, enter a value of  0.

The Slope Shape Factor is an identification number used to indicate the dominant slope shape within the cell.  The slope shape factors are as follows:

1 = uniform slope;

2 = convex slope;

3 = concave slope.

Refer to Figure 3-6.

0FIGURE 3-6.  Examples of various slope shapes

The Field Slope Length is the length of the overland portion of the flow, from the top of the slope to the point where flow becomes concentrated.  This length should not exceed 300 feet.  If the cell is predominantly water or marsh, enter a value of  0.

The Overland Manning's Coefficient is the roughness coefficient for the predominant surface condition within the cell at the time of the storm.  Refer to Table 3-3.  Note that this parameter is identified as the overland Manning's Coefficient.  The user is given the flexibility to input 2 roughness values, one for overland conditions and one for the channel.  The channel roughness value is entered in a pop up screen when the user enters a channel type.

No channel roughness coefficient will be asked for when the user has selected "no discernible channel" or if a water cell exists.  In the case of "no discernible channel", the value that was input for the overland Manning's is used.

Overland Cover and Cover Density                          Overland

Manning's

Coefficient

Smooth, Bare Soil less than 1 inch deep    	  	 0.030

Smooth Bare Soil 1 to 2 inches deep            		 0.033

Smooth Bare Soil, 2-4 inches deep           		 0.038

Smooth Bare Soil, 4-6 inches deep             		 0.045

(Assumes that residue stays in place and does not wash off)

Cornstalks, 1 ton/acre             			0.050

Cornstalks, 2 tons/acre                 		0.075

Cornstalks, 3 tons/acre                                		0.100

Cornstalks, 4 tons/acre                             		0.130

(Assumes that residue stays in place and does not wash off)

Wheat Straw, 1 ton/acre                               	 	0.060

Wheat Straw, 1.5 tons/acre                             	0.100

Wheat Straw, 2 tons/acre                               	0.150

Wheat Straw, 4 tons/acre                                	0.250

(Assumes that grass is erect and as deep as flow)

Grass, sparse                                               	 	0.040

Grass, poor                                                  		0.050

Grass, fair                                                  		0.060

Grass, good                                                  		0.080

Grass, excellent                                            	 	0.130

Grass, dense                                                 		0.200

Grass,very dense                                           	  	0.300

(20% to full maturity, rows with flow)

Small Grain, poor, 7 inch rows                    	 	0.130

Small Grain, poor, 14 inch rows                      	0.130

Small Grain, good, 7 inch rows                       	0.300

Small Grain, good, 14 inch rows                     	0.200

Water or Marsh1                                     	     	0.990

0TABLE 3-3. Overland Manning's Coefficients

Source: Foster et al. (1980)

1  This value serves as a flag only to tell the computer that the surface is water.

The K factor is the Soil Erodibility Factor used in the Universal Soil Loss equation, and is obtained from SCS soils data.  The parameter is the "rate of soil loss per erosion index unit as measured on a unit plot".  The soil erodibility factor will vary with the type of soil that is in each cell.    More information about this parameter can be found in the USDA Agricultural Handbook number 537 "Predicting Rainfall Erosion Losses" (Wischmeier and Smith, 1978).  Values for most of the soil types and textures can be found in tables available from the Regional Technical Service Centers or the State offices of the SCS.  If a cell is predominantly water or marsh, enter a value of  0.

The C factor is the Cropping Management Factor used in the universal soil loss equation.  This parameter is "the ratio of soil loss from land cropped under specified conditions to the corresponding loss from clean tilled continuous fallow.  This ratio also depends on cover, crop sequence, and management practices as well as on the stage of growth of the cover.  Since we are working on a single storm basis, the value to be used is the soil loss ratio corresponding to the appropriate period of the growing season.  In order to keep the analysis consistent between different watersheds, the user will normally examine the worst case condition occurring during the fallow or seedbed periods.  Appropriate soil loss ratio values can be obtained from USDA Agricultural Handbook 537, Predicting Rainfall Erosion Losses (Wischmeier and Smith, 1978).  If the cell is predominantly water or marsh, enter a value of 0.  If the cell is mostly urban or residential, enter a value of 0.01.

The P factor is the conservation Practice Factor or support practice parameter used in the universal soil loss equation.  The practice factor is "the ratio of soil loss with a specific support practice to the corresponding loss with up-and-down-slope culture.  To examine a worst case situation, assume a P-factor of 1.0.  Other values for various conservation practices can be found in USDA Agricultural Handbook 537, on pages 34-39.  Practice factors for terraced fields can be found in table 3-4, below.  If the cell is predominantly water or marsh, enter a value of 0.  If the cell is mostly urban or residential, enter a value of 1.0.

Terrace Grade (%)                       Practice

Factor

Closed outlet or terrace

with underground outlet	  0.052

0% grade (Level)            	  0.10

0.1 % grade                  	  0.13

0.2% grade                    	  0.17

0.4% grade                    	  0.29

0.6% grade                     	  0.49

0.8% grade                     	  0.83

0.9% or steeper grade     	 1.00

0TABLE 3-4.  PRACTICE FACTOR FOR TERRACES

SOURCE: Foster and Highfill

2  Wischmeier and Smith

The Surface Condition Constant is a value based on land use at the time of the storm to make adjustments for overland flow velocity.  Typical values are shown in Table 3-5.

Land Use                        		  	 Surface

Condition

Constant

Fallow                            			 0.22

Row Crop-Straight                  		 0.05

Row Crop-Contoured                 		 0.29

Small Grain                        			 0.29

Legumes or rotation meadow        		 0.29

Pasture-Poor                       			 0.01

Pasture-Fair                       			 0.15

Pasture-Good                       		 0.22

Permanent Meadow                  		 0.59

Woodland                           			 0.29

Forest with heavy litter          		 0.59

Farmsteads                       			 0.01

Urban (21-27%) Impervious         		 0.01

Grass Waterway                    		 1.00

Water                              			 0.00

Marsh                              			 0.00

Animal Lot-Unpaved       		          	 0.00

Animal Lot-Paved                 		 0.00

Roof Area                         			 0.00

0TABLE 3-5.  Surface Condition Constants

SOURCE: Young et al. (1982a)

The Chemical Oxygen Demand factor is a value for the COD concentration in runoff, based on the land use in the cell.  Some COD factors for various land use situations are shown in  table 3-6.   You may enter your own COD factor, or one that is shown in the table.

Land Use                              COD

Factor

(mg/l)

Row Crops                            170

Small grain                           80

Pasture and open                      60

Alfalfa                               20

Forested                              65

Fallow                               115

Farmstead and Urban non-residential   80

Water                                  0

Marsh                                 25

0TABLE 3-6.  COD Factors for various land use

SOURCE: Row crops and fallow, Thompson et al. (1978), Harms et al. (1974); Small Grain and Alfalfa, Harms et al. (1974); Pasture and open land, Crow et al.  (1979), Thompson et al. (1978), Harms et al.       (1974); Forested Land, Timmons et al. (1977), R.A. Young, unpublished data; and farmstead and urban

nonresidential, Weibel (1969).

The Soil Texture input is the major soil texture classification for the cell. The major soil texture can be determined from the texture triangle shown in Figure 3-7, below. The user should look up the correct type of soil in the triangle, then look at Table 3-7 below to get the proper input for AGNPS.  For example, if the cell was predominantly silt loam, then the AGNPS soil type would be silt and we would input a 2, to signify silt.  A fourth soil classification, peat, is not shown on the triangle.  If the cell is water, then enter a zero in the soil texture indicator.

0FIGURE 3-7.  Soil triangle with AGNPS soil types.;

Soil      Soil Type

Texture      Input

Parameter

Water         0

Sand          1

Silt          2

Clay          3

Peat          4

0TABLE 3-7. SOIL TEXTURE INPUT PARAMETERS

When the user selects a soil texture other than water and presses <ENTER> a new menu will pop up, showing the cell number that is being edited, the soil texture, and default values for the nutrient characteristics of the soil type selected.    An example of this menu is shown below, where the user has selected a soil type of silt (2) from the spreadsheet (see Figure 3-8).  Nutrient values and extraction coefficients shown are default values for that soil type and any or all values shown may be edited by the user.

<F1>-Help  <F2>-Menu  <F3>-Load  <F4>-Save  <F5>-Quit  <F6>-Initial Data

Cell Number...........       	     1       	    2      	    3      	 4       	5          6             7

Cell Division.........     	 000     	000     	000       000        000      000         100

Rec. Cell Number......      	     4     	    1     	    7     	  7     	4          5 	            7

Rec. Cell Div.........    	 000   	000  	220        220        000     000         230

Flow Direction........       	     5            7             5            6            7         7            3

SCS Curve Num	                       Cell # 1.000 		               78       82        100

Land Slope (                                             			.1       0.8         0.0

Slope Shape              SOIL INFORMATION - Silt           		 3          1            1

Slope Length                                             		              00      100            0

Overland Man    Soil Nitrogen (lb N/lb soil) .....       0.0010           71    0.130     0.990

K - Factor..        Soil Phosphorus (lb P/lb soil) ...   0.0005            30     0.29        0.00

C - Factor..        Pore Water N Conc. (ppm) .........  5.0000           00  0.4300    0.0000

P - Factor..        Pore Water P Conc. (ppm) .........  2.0000            00     1.00        0.00

Surf Cond Co    N Extraction Coef. for Runoff ....  0.0500            12     0.29        0.00

COD Factor..     P Extraction Coef. for Runoff ....   0.0250           25        80             0

Soil Texture      N Extraction Coef. for Leaching .. 0.2500              2         3             0

Fertilizer            P Extraciton Coef. for Leaching ..0.2500              1         1             0

Pesticide In       Percent Organic Matter in Soil ...  20                     0         0             0

Point Source                                             			  0         0             0

Additional E      <F2> = Default Values      <ESC> = Done               0         0             0

Impoundment						  0         0             0

Channel Indi  						  1         1             0

<F7>-Go To  <F8>-Copy Cell  <F9>-Copy Row  <F10>-Search Row  <AltF1>-Clear

<AltF2>-Delete Cell  <AltF3>-Divide Cell  <AltF4>-Status  <AltF5>-Print

0FIGURE 3-8. Soil Texture pop-up Menu

The Fertilizer Indicator is a value of 0 or 1 indicating whether fertilizer has been applied to the cell or not.  0 indicates no fertilization while 1 indicates fertilization being applied.  If the cell is predominantly water or marsh, enter 0.  When a value of 1 is entered, a pop-up window will appear allowing you to enter either a level of fertilization or the actual amounts applied (see Figure 3-9).